## 3-3 Operation

### 3-3-4 Booting Area Download

1. Turn off the power switch.

2. Connect the serial#1 cable of register to PC.

3. Turn the mode key to ‘SM’-MODE.

4. Press and hold the ‘CASH’ key and ‘Recd Acct’ on key board and turn on the power switch at the same time.

5. The buzzer beeps.

6. DoubleClick ‘DOWNLOAD.EXE’ program on your PC.

7. Select the booting file and PORT.

8. Select the ‘Boot area only’.

9. Click ‘OK’

### 3-3-5 Program Area Download

1. Turn off the power switch.

2. Connect the serial#1 cable of register to PC.

3. Turn the mode key to ‘SM’-MODE.

4. Press and hold the ‘CASH’ key and ‘Recd Acct’ on key board and turn on the power switch at the same time.

5. The buzzer beeps.

6. DoubleClick ‘DOWNLOAD.EXE’ program on your PC.

7. Select the program file and PORT.

8. Select the ‘Program Code only’.

9. Click ‘OK’

10. Wait for about three minutes.

11. Turn off the power switch.

12. Remove the serial cable

13. Perform the all clear function